Project Documentation

Adaptive Cruise Control (ACC) with Longitudinal Vehicle Dynamics – Simulink Model

Objective

To design and simulate an Adaptive Cruise Control (ACC) system that:

- Maintains a driver-set speed (cruise control mode)

- Maintains a safe distance from a lead vehicle (ACC mode)

- Automatically switches between acceleration and braking

Model Type

- 1DOF Longitudinal Vehicle Model

- Force-based dynamics (no lateral dynamics)

Inputs

- Driver Set Speed (via knob + toggle)

- Lead Vehicle Speed

- Initial Distance between vehicles

- Ego Vehicle Speed (from vehicle model feedback)

- Minimum Distance

- Time Gap

Outputs

- Ego Vehicle Speed (xdot)

- Actual Distance

- Safe Distance

- Distance Error

- Acceleration Commands:

  - AccCmdSpeed

  - AccCmdDistance

  - FinalAccCmd

- Wheel Forces:

  - FwF (Front – throttle)

  - FwR (Rear – braking)

Control Logic (Core Concept)

Your system has two controllers:

1️⃣ Speed Controller (Cruise Mode)

- Error:

ev=Vset-Vego

- PID → outputs AccCmdSpeed

- Purpose: maintain desired speed

2️⃣ Distance Controller (ACC Mode)

- Safe distance:

Dsafe=Dmin+Tgap⋅Vego

- Error:

ed=Dactual-Dsafe

- PID → outputs AccCmdDistance

- Purpose: avoid collision

3️⃣ Decision Logic (MOST IMPORTANT)

You used:

FinalAccCmd=min⁡(AccCmdSpeed,  AccCmdDistance)

✅ Why MIN works (Physics Insight)

- AccCmdSpeed → usually positive (accelerate)

- AccCmdDistance → becomes negative (brake) when too close

So:

- If safe → speed controller dominates

- If unsafe → distance controller dominates

✔ This is correct real-world ACC logic

Vehicle Dynamics

Force → Acceleration:

F=m⋅a

Then:

- Acceleration → Velocity (xdot)

- Velocity → Distance (via integrator)

Workflow (Step-by-Step)

1. Driver Input

- Knob → desired speed

- Toggle:

  - 0 → Manual mode

  - 1 → ACC enabled

2. Set Speed Capture

- When toggle = 1 → current speed is latched

- This becomes SetSpeed

3. Speed Control Loop

- Compare:

SetSpeed - EgoSpeed

- PID → AccCmdSpeed

4. Distance Calculation

- Lead & Ego distances via integration

- Compute:

ActualDistance = LeadDistance - EgoDistance

5. Safe Distance Calculation

SafeDistance = MinimumDistance + TimeGap × EgoSpeed

6. Distance Control Loop

- Error:

DistanceError = ActualDistance - SafeDistance

- PID → AccCmdDistance

7. Decision Block

FinalAccCmd = min(AccCmdSpeed, AccCmdDistance)

8. Force Distribution

- If FinalAccCmd > 0 → throttle → FwF

- If FinalAccCmd < 0 → brake → FwR

9. Vehicle Response

- Force → acceleration → velocity → position

Results (Graphs Analysis)

1️⃣ Ego Speed (xdot)

- Increases linearly → indicates constant acceleration

- No saturation → missing drag/limits

2️⃣ Distance Error

- Increasing → ego not closing gap properly

- Means:

  - Either lead is faster

  - Or controller not aggressive enough

3️⃣ Safe Distance

- Linear increase → correct (depends on speed)

4️⃣ AccCmdSpeed

- Near zero → system reached steady state

5️⃣ AccCmdDistance

- Increasing → distance controller trying to act

6️⃣ FinalAccCmd

- Same as speed command most of time

7️⃣ Wheel Forces

- FwF active → acceleration

- FwR mostly zero → no braking